1.	Handwheel over back left corner of area, approximately 0.50 over part.

2.	Measure X and Y area of probing area.

3.	Call with:

G541 X[length] Y[depth] (S[offset])

X = Overall width of area to be measured

Y = Overall depth of area to be measured

S = Optional, defaults to G54

4.	Cycle will end with probe over highest point, setting that spot as Z=0

Max point spacing = 4.000”, cycle will automatically add points as needed.

Example:

G541 X10.0 Y8.0